A					      B					C

NAT

Private IP- 10.0.1.1

Public IP – 63.143.15.129

Internet

PC – IP Addr -10.0.1.10

Netmeeting running here connected to somebody on Internet

A PC –Pentium

B Router NAT enabled

C internet service/modem/DSL etc.

SCENE – I  ------ Doesnot work -- watch status of B -- NAT enabled

A					      B					C

Bridge

IP – 63.143.15.129

Internet

PC – IP Addr -63.143.15.120

Netmeeting running here connected to somebody on Internet

A PC –Pentium

B Same Router NAT disabled. Bridging turned ON

C internet service/modem/DSL etc.

SCENE – II  ------ work -- watch status of B -- NAT Disabled. Straight bridging

Internet

# A					B		C

PC –pentium

IP-192.168.1.2

PC –pentium

IP-192.168.1.1

Public IP – 63.143.15.129

A PC –Pentium

B PC –Pentium – DHCP turned on. Pvt. IP 192.168.1.1 i.e. Ethernet side. Modem interface has DHCP enabled from the Internet Service provider (ISP) and receives a public IP of say 63.143.15.129.

Here B is doing an Internet connection sharing. A cross over cable or hub can be used to do this. This looks more like a NAT case as the IP space of the private side is changing.

C internet service/modem/DSL etc.

SCENE – III--- doesnot work -- watch status of B – Using internet sharing by Microsoft.

The most interesting part of all of the above is that the traffic passes through the private side of NAT to the public side but no traffic from public to the private side.

I mean the person sitting at C can hear what A says and see him as well but A receives no data from C. He cannot see or listen whatever C says.

This happens in case of SCENE III and I.

In SCENE II everything works fine …… A NAT is supposed to pass all IP packets and all packets in Netmeeting are IP packets especially UDP and TCP (RTP as well which is over UDP). A bridge is supposed to bridge all traffic. This raises a question whether there is any NON IP traffic in handshake of Netmeeting????

In SCENE III netmeeting between B and C works just fine. But not between A and C or A and B. Well the combination “A-C” and “A-B” mean almost the same setup here just that A-B is a little more complicated than A-C as far as netmeeting is concerned.