# NASA NEWS

Space Shuttle Processing Status Report

## Contact:  Jessica Rye									Vol. 1   No. 3

(321) 867-6185

### Space Shuttle Processing Status Report

Friday, March 12, 2004 (11 a.m.)

Note: NASA’s Kennedy Space Center issues Space Shuttle Processing Status Reports each week, and is the source for information regarding processing activities associated with the vehicles and payloads. This report does not necessarily reflect the chronological order of future Space Shuttle missions.  If you are a member of the media and would like further information, visit http://www- pao.ksc.nasa.gov/kscpao.htm

Discovery (OV-103)

At Kennedy Space Center, Discovery continues to be processed for the Return to Flight mission. The four Rudder Speed Brake actuators were X-rayed on Monday and Tuesday to determine if the gears were installed correctly. One of the four actuators, actuator No. 2, showed that a gear was reversed. It will be sent back to the vendor for refurbishment, and returned to KSC for re-installation on the vehicle.

Vehicle power-up testing continues with the Main Propulsion System leak tests and fuel cell system checkout. The Body Flap actuator fit check was successfully completed. Build-up of Reinforced Carbon-Carbon (RCC) panels and associated fittings is ongoing. Six RCC spar fittings have been installed on the left-hand leading edge.

Atlantis (OV-104)

Preparations continue in the processing facility for Atlantis’ future mission to the International Space Station.  Seven T-seals, the C-shaped seals that fit between each RCC panel, have been installed on the left-hand wing leading edge. Build-up of the right-hand RCC panels and associated fittings is ongoing.

The nose cap was delivered to the Thermal Protection System Facility (TPSF) on Tuesday. Technicians in the TPSF are beginning to build the thermal blankets that will be installed in the nose cap. Installation of window No. 2 is scheduled for Friday.

Endeavour (OV-105)

Space Shuttle Endeavour is in its Orbiter Major Modification period, which began in December 2003.

Thermal Protection System blankets have been removed from the Rudder Speed Brake panels in preparation for the removal and inspection of the actuators. RCC panels continue to be removed from the vehicle and returned to the vendor for inspection.

--end--

The Kennedy Space Center Newsroom offers an electronic subscription service for status reports, news releases and other notices issued from KSC. There are two possible ways to subscribe.  You may send a blank e-mail message to ksc-news_release-subscribe@kscnews.ksc.nasa.gov or follow the instructions on the Web site at http://kscnews.ksc.nasa.gov. The system will confirm the request via e-mail.