# NASA NEWS

Space Shuttle Processing Status Report

## Contact:  Jessica Rye									Vol. 1   No. 17

(321) 867-6185

### Space Shuttle Processing Status Report

Friday, June 18, 2004 (12:00 p.m.)

Note: NASA’s Kennedy Space Center issues Space Shuttle Processing Status Reports each week, and is the source for information regarding processing activities associated with the vehicles and payloads. This report does not necessarily reflect the chronological order of future Space Shuttle missions.  If you are a member of the media and would like further information, visit http://www-pao.ksc.nasa.gov/kscpao.htm

Discovery (OV-103)

Discovery’s month-long power-down period continues for its Return to Flight mission, STS-114, to the International Space Station. The Forward Reaction Control System was moved Wednesday from the Hypergolic Maintenance Facility to the Orbiter Processing Facility. Installation on the vehicle is scheduled for tomorrow morning.

Return to Flight modifications continue, including the orbiter boom sensor system wiring and preparations for installation of the wing leading edge sensors. Both Freon Coolant Loop No. 1 and 2 have been drained, and water coolant loop No. 2 leak checks are in work.

Atlantis (OV-104)

Atlantis’ four-month power-down period continues in the OPF in support of Return to Flight modifications, as well as structural and baseline wire inspections. On Wednesday, right-hand radiator No. 3 and 4 were removed from the vehicle.

Left-hand Reinforced Carbon-Carbon panels No. 19 and 20 have been removed to support the addition of wing leading edge sensor instrumentation. The wing leading edge sensors that are being installed will allow mission managers to receive data that will help to determine any impacts to the wing during launch.

Endeavour (OV-105)

Space Shuttle Endeavour is in its Orbiter Major Modification period, which began last December. Electrical modifications continue in the crew module. Wire inspections are ongoing in the Environmental Control and Life Support System bay.

Installation of left-hand radiator No. 1 is scheduled for today, with the No. 2 radiator scheduled for installation early next week. Flex hose inspections continued throughout the week.

-end-

The Kennedy Space Center Newsroom offers an electronic subscription service for status reports, news releases and other notices issued from KSC. There are two possible ways to subscribe.  You may send a blank e-mail message to ksc-news_release-subscribe@kscnews.ksc.nasa.gov or follow the instructions on the Web site at http://kscnews.ksc.nasa.gov. The system will confirm the request via e-mail.